Dataset Preprocessing Procedures

This document summarizes the preprocessing procedures used for the UCI Condition Monitoring of Hydraulic Systems dataset and the NASA B0006 lithium-ion battery dataset.

1. UCI Condition Monitoring of Hydraulic Systems Dataset

Sample construction. Each operating cycle was treated as one sample. The PS2 pressure signal within each cycle was used for feature extraction.

Feature extraction. Two statistical features were extracted from the PS2 pressure signal: skewness (PS) and kurtosis (PK). Bias-corrected sample skewness was used for PS, and bias-corrected kurtosis was used for PK.

Health-state labeling. According to the original valve-condition labels, 100%, 90%, 80%, and 73% were mapped to healthy (H), slight degradation (G), moderate degradation (M), and failure (D), respectively.

Data partition. A total of 401 samples were used, including 206 H samples and 65 samples for each of G, M, and D. Stratified sampling with a fixed random seed of 2039 was used, and the dataset was split into training and test sets at a ratio of 7:3. The specific operating-cycle indices used in the experiment are provided in the supplementary material.

2. NASA B0006 Lithium-Ion Battery Dataset

Battery selection and health indicator. The B0006 lithium-ion battery was selected for evaluation. Battery capacity was adopted as the health indicator (HI) to characterize the degradation state.

Input-feature extraction. Two charging-related features were extracted as model inputs: constant-current charging time (PC-time-CC) and constant-voltage charging time (PC-time-CV). PC-time-CC represents the duration of the constant-current charging stage, whereas PC-time-CV represents the duration of the constant-voltage charging stage.

Sample construction. Each charge-discharge cycle was treated as one sample. The two charging-time features were used as input attributes, and the corresponding discharge capacity was used as the health indicator.

Dataset size. A total of 166 samples were used in the experiment, with 116 samples for training and 50 samples for testing.

3. Summary

| Dataset | Sample unit | Input features | Target / HI |
| --- | --- | --- | --- |
| UCI Hydraulic | One operating cycle | PS2 skewness (PS), PS2 kurtosis (PK) | Four valve-health states: H/G/M/D |
| NASA B0006 | One charge-discharge cycle | PC-time-CC, PC-time-CV | Battery capacity |